Ford KUGA Specifications

FUEL CONSUMPTION and Economy

| Petrol engines |  |  | CO2 from (g/km NEDC) | Fuel consumption from (l/100 km NEDC) |  |  | Fuel consumption from (l/100 km WLTP) | CO2 from (g/km WLTP) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Power PS | Wheel size inch | Combined | Urban | Extra Urban | Combined | Overall | Overall |
| 1.5-litre EcoBoost 6-speed manual | 120 | 17 | 127 | 6.8 | 4.9 | 5.6 | 6.7 | 152 |
|  |  | 18 | 130 | 6.9 | 5.1 | 5.8 | 6.9 | 156 |
|  |  | 19 | 130 | 6.9 | 5.1 | 5.8 | 6.9 | 156 |
| 1.5-litre EcoBoost 6-speed manual | 150 | 17 | 125 | 6.5 | 4.9 | 5.5 | 6.6 | 150 |
|  |  | 18 | 129 | 6.7 | 5.1 | 5.7 | 6.8 | 155 |
|  |  | 19 | 129 | 6.7 | 5.1 | 5.7 | 6.8 | 155 |
|  |  | 20 | 129 | 6.7 | 5.1 | 5.7 | 6.8 | 155 |

| Diesel engines |  |  | CO2 from (g/km NEDC) | Fuel consumption from (l/100 km NEDC) |  |  | Fuel consumption from (l/100 km WLTP) | CO2 from (g/km WLTP) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Power PS | Wheel size inch | Combined | Urban | Extra Urban | Combined | Overall | Overall |
| 1.5-litre EcoBlue 6-speed manual | 120 | 17 | 105 | 4.4 | 3.7 | 4.0 | 4.7 | 123 |
|  |  | 18 | 106 | 4.5 | 3.8 | 4.1 | 4.8 | 126 |
|  |  | 19 | 106 | 4.5 | 3.8 | 4.1 | 4.9 | 127 |
|  |  | 20 | 106 | 4.5 | 3.8 | 4.1 | 4.8 | 126 |
| 1.5-litre EcoBlue 8-speed auto | 120 | 17 | 117 | 5.1 | 4.0 | 4.4 | 5.2 | 137 |
|  |  | 18 | 118 | 5.1 | 4.1 | 4.5 | 5.3 | 140 |
|  |  | 19 | 118 | 5.1 | 4.1 | 4.5 | 5.4 | 142 |
|  |  | 20 | 118 | 5.1 | 4.1 | 4.5 | 5.4 | 140 |
| 2.0-litre EcoBlue 8-speed auto | 190 | 17 | 126 | 5.6 | 4.4 | 4.8 | 5.7 | 150 |
|  |  | 18 | 128 | 5.7 | 4.4 | 4.9 | 5.8 | 153 |
|  |  | 19 | 128 | 5.7 | 4.4 | 4.9 | 5.9 | 154 |
|  |  | 20 | 128 | 5.7 | 4.4 | 4.9 | 5.9 | 153 |

| Hybrid engines |  |  | CO2 from (g/km NEDC) | Fuel consumption from (l/100 km NEDC) |  |  |  | Fuel consumption from (l/100 km WLTP) | CO2 from (g/km WLTP) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Power PS | Wheel size inch | Combined | Urban | Extra Urban | Combined |  | Overall | Overall |
| 2.0-litre EcoBlue Hybrid 6-speed manual | 150 | 17 | 109 | 4.3 | 4.0 | 4.1 |  | 4.9 | 127 |
|  |  | 18 | 112 | 4.5 | 4.1 | 4.3 |  | 5.0 | 131 |
|  |  | 19 | 112 | 4.5 | 4.1 | 4.3 |  | 5.1 | 132 |
|  |  | 20 | 112 | 4.5 | 4.1 | 4.3 |  | 5.0 | 131 |
| 2.5-litre Petrol Hybrid | 190 | 17 | 118 | 6.3 | 4.5 | 5.1 |  | 5.4 | 125 |
|  |  | 18 | 119 | 6.4 | 4.5 | 5.2 |  | 5.6 | 129 |
|  |  | 19 | 119 | 6.4 | 4.5 | 5.2 |  | 5.7 | 131 |
|  |  | 20 | 119 | 6.4 | 4.5 | 5.2 |  | 5.7 | 129 |
|  |  | 17 | 26 | N/A | N/A | 1.2 |  | 1.4 | 32 |
| 2.5-litre Petrol Plug-In Hybrid | 225 | 18 | 26 | N/A | N/A | 1.2 |  | 1.4 | 32 |
|  |  | 19 | 26 | N/A | N/A | 1.2 |  | 1.4 | 32 |
|  |  | 20 | 26 | N/A | N/A | 1.2 |  | 1.4 | 32 |
|  |  | Electric range NEDC |  |  |  |  | 72 km (44.7 miles) |  |  |
|  |  | Equivalent all-electric range overall WLTP |  |  |  |  | 56 km (34.8 miles) |  |  |
|  |  | Equivalent all-electric range city WLTP |  |  |  |  | 68 km (42.2 miles) |  |  |

PERFORMANCE

| Petrol engines | Power PS | Max speed km/h (mph) | 0-100 km/h sec | 50-100 km/h sec1 |
| --- | --- | --- | --- | --- |
| 1.5-litre EcoBoost 6-speed manual | 120 | 184 (114) | 11.6 | 9.5 |
| 1.5-litre EcoBoost 6-speed manual | 150 | 195 (121) | 9.7 | 9.5 |

| Diesel engines | Power PS | Max speed km/h | 0-100 km/h sec | 50-100 km/h sec1 |
| --- | --- | --- | --- | --- |
| 1.5-litre EcoBlue 6-speed manual | 120 | 180 (112) | 11.7 | 11.5 |
| 1.5-litre EcoBlue 8-speed auto | 120 | 177 (110) | 12 | N/A |
| 2.0-litre EcoBlue 8-speed auto | 190 | 208 (129) | 8.7 | N/A |

| Hybrid engines | Power PS | Max speed km/h | 0-100 km/h sec | 50-100 km/h sec1 |
| --- | --- | --- | --- | --- |
| 2.0-litre EcoBlue Hybrid 6-speed manual | 150 | 194 (121) | 9.6 | 7.8 |
| 2.5-litre Petrol Hybrid | 190 | 196 (122) | 9.1 | N/A |
| 2.5-litre Petrol Plug-In Hybrid | 225 | 200 (124) | 9.2 | N/A |

1 In 4th gear

WEIGHTS

|  | Kerb weight (kg)2 | Gross Vehicle Mass (kg) | Gross Train Mass (kg) | Max. Towable Mass (braked) (kg) | Max. Towable Mass (unbraked) (kg) | Max. Roof Load (kg) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.5-litre EcoBoost (120 PS) 6-speed manual FWD | 1564 | 2075 | 3675 | 1600 | 750 | 75 |
| 1.5-litre EcoBoost (150 PS) 6-speed manual FWD | 1564 | 2075 | 3875 | 1800 | 750 | 75 |
| 1.5-litre EcoBlue (120 PS) 6-speed manual FWD | 1576 | 2085 | 3585 | 1500 | 750 | 75 |
| 1.5-litre EcoBlue (120 PS) 8-speed auto FWD | 1599 | 2110 | 3610 | 1500 | 750 | 75 |
| 2.0-litre EcoBlue (190 PS) 8-speed auto AWD | 1735 | 2245 | 4345 | 2100 | 750 | 75 |
| 2.0-litre EcoBlue Hybrid (150 PS) 6-speed manual FWD | 1680 | 2155 | 4055 | 1900 | 750 | 75 |
| 2.5-litre Petrol Hybrid (190 PS) FWD | 1701 | 2215 | 3815 | 1600 | 750 | 75 |
| 2.5-litre Petrol Hybrid (190 PS) AWD | 1773 | 2280 | 3780 | 1500 | 750 | 75 |
| 2.5-litre Petrol Plug-In Hybrid (225 PS) | 1844 | 2320 | 3520 | 1200 | 750 | 75 |

2 Represents the lightest kerbweight assuming driver at 75 kg, full fluid levels and 90 per cent fuel levels, subject to manufacturing tolerances and options, etc., fitted. Weights represent base model specification without panoramic glass roof.

Towing limits quoted represent the maximum towing ability of the vehicle at its Gross Vehicle Mass to restart on a 12 per cent gradient at sea level. The performance and economy of all models will be reduced when used for towing.  Gross Train Mass includes trailer weight.

DIMENSIONS3

|  |  | Kuga Trend and Titanium | Kuga Vignale | Kuga ST-Line | Kuga Hybrid and Kuga Plug-In Hybrid (Trend and Titanium) |
| --- | --- | --- | --- | --- | --- |
| - Overall length | mm | 4614 | 4629 | 4626 | 4614 |
| Overall width with mirrors | mm | 2178 | 2178 | 2178 | 2178 |
| Overall width without mirrors | mm | 1883 | 1883 | 1883 | 1883 |
| Overall width with mirrors folded | mm | 2000 | 2000 | 2000 | 2000 |
| Overall height with roof rails | mm | 1678 | 1680 | 1666 | 1675 |
| Wheelbase | mm | 2710 | 2710 | 2710 | 2710 |
| - Track front | mm | 1597 | 1597 | 1599 | 1597 |
| - Track rear | mm | 1588 | 1588 | 1590 | 1588 |
| - Approach angle | deg | 19.1 | 19.3 | 18.0 | 18.8 |
| - Departure angle | deg | 23.6 | 23.5 | 22.6 | 24.4 |
| - Minimum running ground clearance - kerb | mm | 191 | 190 | 178 | 160 |
| - Ground clearance at maximum load | mm | 146 | 148 | 137 | 139 |
| - Interior First Row |  |  |  |  |  |
| - Headroom without sunroof | mm | 1017 | 1017 | 1017 | 1017 |
| - Headroom with sunroof | mm | 985 | 985 | 985 | 985 |
| Legroom | mm | 1077 | 1077 | 1077 | 1077 |
| Shoulder room | mm | 1464 | 1464 | 1464 | 1464 |
| Hip room | mm | 1410 | 1410 | 1410 | 1410 |
| Interior Second Row |  |  |  |  |  |
| Headroom without sunroof | mm | 999 | 999 | 999 | 999 |
| - Headroom with sunroof | mm | 981 | 981 | 981 | 981 |
| - Minimum leg room | mm | 986 | 986 | 986 | 942 |
| - Maximum leg room | mm | 1035 | 1035 | 1035 | 989 |
| - Shoulder room | mm | 1423 | 1423 | 1423 | 1423 |
| - Hip room | mm | 1355 | 1355 | 1355 | 1355 |
| - Cargo Compartment |  |  |  |  |  |
| - Lift in height | mm | 715 | 713 | 702 | 712 |
| - Cargo height | mm | 885 | 855 | 855 | 833 |
| - Rear opening height | mm | 773 | 773 | 773 | 773 |
| - Loading length at floor to first row | mm | 1739 | 1739 | 1739 | 1729 |
| - Load width between wheel arches | mm | 1051 | 1051 | 1051 | 1051 |
| - Load opening width at floor | mm | 1070 | 1070 | 1070 | 1070 |
| - Luggage capacity |  |  |  |  |  |
| - Cargo volume behind first row (2-seat mode; laden to roof) with tyre repair kit4 | litres | 1534 | 1534 | 1534 | 1481 |
| - Cargo length at floor behind second row with tyre repair kit | mm | 961 | 961 | 961 | 933 |
| - Closed cargo compartment second row (under cargo shade) with tyre repair kit (min-max depending on sliding second row position) | litres | 475-645 | 475-645 | 475-645 | 411-581 |

3 Front-wheel drive with 17- to 19-inch wheels

4 V214-1. Dimensions may vary dependent on the model and equipment fitted.

DRIVER ASSISTANCE TECHNOLOGIES5

Active Park Assist 2

Adaptive Cruise Control with Stop & Go, Speed Sign Recognition and Lane Centring

Adaptive Front Lighting System (including predictive curve light and sign-based light)

Auto high beam

Auto lighting

Blind Spot Information System (BLIS)

Cross Traffic Alert

Distance Indication

Driver Alert

Emergency Brake Assist

Evasive Steering Assist

Forward Collision Warning with Brake Support

Front and rear parking sensors

Front wide-view camera

Glare-free high beam

Hill Start Assist

Intelligent Speed Limiter

Intersection Assist 1.0

Enhanced Lane-Keeping System with Road Edge Detection and BLIS Assist

Local Hazard Information

Post-Collision Braking

Pre-Collision Assist with Active Braking

Rain-sensing wipers

Rear wide-view camera

Selectable Drive Modes

Traffic Sign Recognition

Wrong Way Alert

COMFORT AND CONVENIENCE5

12.3-inch LCD instrument cluster

B&O Sound System

FordPass Connect modem

Hands-free tailgate

Head-up display

Openable panoramic roof

Retractable tow bar

SYNC 3 with 8-inch touchscreen

Wireless charging pad

5Feature availability dependent on vehicle specification

STEERING

| System | Column-mounted electric power-assisted steering, mechanical steering gear |
| --- | --- |
| Ratio | 13.75:1 |
| Turning circle (m) kerb-to-kerb | 11.38 |

CHASSIS

| Front suspension | Independent MacPherson struts, L-shaped lower control arm, steering gear and hollow stabilizer bar mounted on subframe |
| --- | --- |
| Rear suspension | Isolated subframe for the Short Long Arm (SLA) independent rear suspension, multi-compound bushes with different stiffness characteristics separating the subframe from the body structure, springs located below floor, hollow stabilizer. |

BRAKES

|  | Front | Rear |
| --- | --- | --- |
| Braking | Hydraulically operated dual-circuit system with diagonal distribution. Vented front discs. Solid rear discs with Electric Park Brake system (EPB). Electronic four-channel anti-lock braking system (ABS) with electronic brake-force distribution (EBD). Electronic brake booster on hybrid powertrains and for selected driver assistance technologies. Electronic stability control system (ESC), including roll stability control (RSC) and trailer sway control (TSC). |  |
| Disc dimensions (mm) | Ø308 x 27 Ø330 x 27 | Ø302 x 11 |
| Piston dimensions (mm) | Ø57 2 x Ø44 | Ø36 (iEPB) |

WHEELS & TYRES

7 x 17-inch with 225/65 R17

7.5 x 18-inch with 225/60 R18

7.5 x 19-inch with 225/55 R19

7.5 x 20-inch with 245/45 R20

8.0 x 20-inch with 245/45 R20

PETROL ENGINES

|  |  | 1.5-litre EcoBoost (120, 150 PS) |  |
| --- | --- | --- | --- |
| Type |  | In-line three-cylinder turbo petrol, Ti-VCT, transverse |  |
| Displacement | cm3 | 1497 |  |
| Bore | mm | 84.0 |  |
| Stroke | mm | 90.0 |  |
| Compression ratio |  | 11.0:1 |  |
| Max power | PS (kW) | 120 (88) | 150 (110) |
|  | at rpm | 5500 | 6000 |
| Max torque steady state | Nm | 240 | 240 |
|  | at rpm | 1600-3000 | 1600-4000 |
| Valve gear |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |  |
| Cylinders |  | 3 in-line, cylinder deactivation technology on cylinder one |  |
| Cylinder head |  | Cast aluminium with iron liners |  |
| Cylinder block |  | Cast aluminium |  |
| Camshaft drive |  | Chain driven with hydraulic tensioner |  |
| Crankshaft bearings |  | Cast iron, 6 counterweights, 4 main bearings |  |
| Engine management |  | Bosch MG1CS016 with CAN-Bus and individual cylinder knock control. FGEC software |  |
| Fuel injection |  | 3 high-pressure direct fuel injection, 6-hole injectors (DI) plus 3 low-pressure port injectors (PFI) |  |
| Emission level |  | Euro 6d-TEMP |  |
| Emission control |  | Rapid light-off catalyst, gasoline particulate filter |  |
| Turbocharger |  | Continental radial-axial (RAAX) turbocharger |  |
| Lubrication system |  | Two-stage variable displacement oil pump |  |
| Cooling system |  | Single thermostat |  |
| Transmission |  | 6-speed manual |  |
| Gear ratios |  | 6th 0.578 5th 0.690 4th 0.868 3rd 1.194 2nd 1.952 1st 3.727 R 3.886 FDR 4.056 |  |

DIESEL ENGINES

|  |  | 1.5-litre EcoBlue (120 PS) |  |
| --- | --- | --- | --- |
| Type |  | In-line four-cylinder turbo diesel, transverse |  |
| Displacement | cm3 | 1499 |  |
| Bore | mm | 75.0 |  |
| Stroke | mm | 84.8 |  |
| Compression ratio |  | 16.4:1 |  |
| Max power | PS (kW) | 120 (88) |  |
|  | at rpm | 4000 |  |
| Max torque steady state | Nm | 300 |  |
|  | at rpm | 1750-2250 |  |
| Valve gear |  | DOHC with 4 valves per cylinder |  |
| Cylinders |  | 4 in-line |  |
| Cylinder head |  | Cast aluminium |  |
| Cylinder block |  | Cast aluminium |  |
| Camshaft drive |  | Low-friction, Kevlar-reinforced primary drive belt. Cam-to-cam chain |  |
| Crankshaft |  | Steel forging, 4 counter-weights, 5 main bearings |  |
| Engine management |  | Bosch FDEC |  |
| Fuel injection |  | Common rail direct fuel injection; 2000 bar injection pressure; 8-hole injectors |  |
| Emission level |  | Euro 6d-TEMP |  |
| Emission control |  | High-pressure exhaust gas recirculation; water-air charge cooling |  |
| Turbocharger |  | Low-inertia variable geometry turbocharger with electronic control |  |
| Lubrication system |  | Pressure-fed lubrication system with full flow oil filter, variable displacement oil pump |  |
| Cooling system |  | Water pump with electrically switchable thermostat |  |
| Transmission |  | 6-speed manual | 8-speed automatic |
| Gear ratios |  | 6th 0.578 5th 0.690 4th 0.868 3rd 1.194 2nd 1.952 1st 3.727 R 3.886 FDR 4.056 | 8th 0.62 7th 0.75 6th 1.00 5th 1.45 4th 1.92 3rd 3.01 2nd 3.31 1st 4.69 R 2.96 FDR 3.47 |

|  |  | 2.0-litre EcoBlue (190 PS) |
| --- | --- | --- |
| Type |  | In-line four-cylinder turbo diesel, transverse |
| Displacement | cm3 | 1995 |
| Bore | mm | 84.0 |
| Stroke | mm | 90.0 |
| Compression ratio |  | 16.5:1 |
| Max power | PS (kW) | 190 (140) |
|  | at rpm | 3500 |
| Max torque steady state | Nm | 400 |
|  | at rpm | 2000-3000 |
| Valve gear |  | DOHC with 4 valves per cylinder |
| Cylinders |  | 4 in-line |
| Cylinder head |  | Cast aluminium |
| Cylinder block |  | Cast iron |
| Camshaft drive |  | Low-friction belt-in-oil with dynamic tensioner |
| Crankshaft |  | Forged steel, 4 counter-weights, 5 main bearings |
| Engine management |  | Ford Common Rail Diesel Engine Management System |
| Fuel injection |  | Common rail direct fuel injection; 2200 bar injection pressure; 8-hole piezo-electric injectors |
| Emission level |  | Euro 6d-TEMP |
| Emission control |  | High- and low-pressure exhaust gas recirculation; water-air charge cooling; selective catalytic reduction after treatment |
| Turbocharger |  | Low-inertia variable geometry turbocharger with electronic control |
| Lubrication system |  | Pressure-fed lubrication system with full flow oil filter, variable displacement oil pump |
| Cooling system |  | Engine driven water pump with thermostat control |
| Transmission |  | 8-speed automatic |
| Gear ratios |  | 8th 0.62 7th 0.75 6th 1.00 5th 1.45 4th 1.92 3rd 3.01 2nd 3.31 1st 4.69 R 2.96 FDR 3.17 |

HYBRID ENGINES

|  |  | 2.0-litre EcoBlue Hybrid (150 PS) |
| --- | --- | --- |
| Type |  | Inline four cylinder turbo diesel, transverse |
| Displacement | cm3 | 1995 |
| Bore | mm | 84.0 |
| Stroke | mm | 90.0 |
| Compression ratio |  | 16.5:1 |
| Max power | PS (kW) | 150 (110) |
|  | at rpm | 3500 |
| Max torque | Nm | 370 |
|  | at rpm | 2000-2500 |
| Valve gear |  | DOHC with 4 valves per cylinder |
| Cylinders |  | 4 inline |
| Cylinder head |  | Cast aluminium |
| Cylinder block |  | Cast iron |
| Camshaft drive |  | Low-friction belt-in-oil with dynamic tensioner |
| Crankshaft |  | Forged steel, 4 counter-weights, 5 main bearings |
| Engine management |  | Ford Common Rail Diesel Engine Management System |
| Fuel injection |  | Common rail direct fuel injection; 2200 bar injection pressure; 8-hole piezo-electric injectors |
| Emission level |  | Euro 6d-TEMP |
| Emission control |  | High- and low-pressure exhaust gas recirculation; water-air charge cooling; selective catalytic reduction after treatment |
| Turbocharger |  | Low-inertia variable geometry turbocharger with electronic control |
| Electric motor |  | 11.5kW belt-driven integrated starter/generator |
| Battery |  | 48V 10Ah lithium-ion air-cooled battery pack |
| Lubrication system |  | Pressure-fed lubrication system with full flow oil filter, variable displacement oil pump |
| Cooling system |  | Engine driven water pump with thermostat control |
| Transmission |  | 6-speed manual |
| Gear ratios |  | 6th 0.526 5th 0.633 4th 0.816 3rd 1.156 2nd 1.864 1st 3.583 R 3.643 FDR 4.533 |

|  |  | 2.5-litre Petrol Hybrid (190 PS) |
| --- | --- | --- |
| Type |  | Atkinson Cycle port fuel injection engine |
| Motor |  | Permanent magnet 3-phase AC synchronous traction motor, liquid cooled |
| Max power | PS (kW) | 190 (140) |
|  | at rpm | 5500 |
| Max torque | Nm | N/A |
|  | at rpm | N/A |
| Battery |  | Lithium-ion |
| High-voltage cooling system |  | Liquid-cooled |
| Battery capacity | kWh | 1.1 kWh |
| Transmission |  | Ford-developed power-split automatic with continuously variable ratio |
| Gear ratio |  | 2.91 FDR |
| Fuel tank | litres | 54 |
| Displacement | cm3 | 2488 |
| Bore | mm | 89.0 |
| Stroke | mm | 100.0 |
| Compression ratio |  | 13.0:1 |
| Valve gear |  | DOHC with 4 valves per cylinder, variable cam timing |
| Cylinders |  | 4 inline |
| Cylinder head |  | Aluminium |
| Cylinder block |  | Aluminium |
| Camshaft drive |  | Chain-in-oil with dynamic tensioner |
| Crankshaft |  | Forged steel, 8 counter-weights, 5 main bearings |
| Engine management |  | Bosch MED17 with CAN-Bus and individual cylinder knock control. Bosch software |
| Fuel injection |  | Multi-port fuel injection |
| Emission level |  | Euro 6d-TEMP |
| Lubrication system |  | Electronically controlled variable displacement oil pump |
| Engine cooling system |  | Liquid cooled, separate from high-voltage cooling/heating system |

|  |  | 2.5-litre Petrol Plug-In Hybrid (225 PS) |
| --- | --- | --- |
| Type |  | Atkinson Cycle port fuel injection engine |
| Motor |  | Permanent magnet 3-phase AC synchronous traction motor, liquid cooled |
| Max power | PS (kW) | 225 (165) |
|  | at rpm | N/A |
| Max torque | Nm | N/A |
|  | at rpm | N/A |
| Battery |  | Lithium-ion |
| High-voltage cooling system |  | Liquid-cooled/heated |
| Battery capacity | kWh | 14.4 kWh 48 Ah |
| Transmission |  | Ford-developed power-split automatic with continuously variable ratio |
| Gear ratio |  | 2.91 FDR |
| Fuel tank | litres | 42.7 |
| Displacement | cm3 | 2488 |
| Bore | mm | 89.0 |
| Stroke | mm | 100.0 |
| Compression ratio |  | 13.0:1 |
| Valve gear |  | DOHC with 4 valves per cylinder, variable cam timing |
| Cylinders |  | 4 inline |
| Cylinder head |  | Aluminium |
| Cylinder block |  | Aluminium |
| Camshaft drive |  | Chain-in-oil with dynamic tensioner |
| Crankshaft |  | Forged steel, 8 counter-weights, 5 main bearings |
| Engine management |  | Bosch MED17 with CAN-Bus and individual cylinder knock control. Bosch software |
| Fuel injection |  | Multi-port fuel injection |
| Emission level |  | Euro 6d-TEMP |
| Lubrication system |  | Electronically controlled variable displacement oil pump |
| Engine cooling system |  | Liquid cooled, separate from high-voltage cooling/heating system |

### DRIVE LINE

| Drive options: | AWD: Electronically controlled Ford Intelligent All Wheel Drive with variable torque distribution |  |  |  |
| --- | --- | --- | --- | --- |
|  | FWD: Front-wheel drive |  |  |  |

# # #

Note: The data information in this press release reflects preliminary specifications and was correct at the time of going to print. However, Ford policy is one of continuous product improvement. The right is reserved to change these details at any time.

The declared fuel/energy consumptions, CO2-emissions and electric range are determined according to the technical requirements and specifications of the European Regulations (EC) 715/2007 and (EU) 2017/1151 as last amended. The applied standard test procedures enable comparison between different vehicle types and different manufacturers.

About Ford Motor Company

Ford Motor Company is a global company based in Dearborn, Michigan. The company designs, manufactures, markets and services a full line of Ford cars, trucks, SUVs, electrified vehicles and Lincoln luxury vehicles, provides financial services through Ford Motor Credit Company and is pursuing leadership positions in electrification; mobility solutions, including self-driving services; and connected services.  Ford employs approximately 187,000 people worldwide. For more information regarding Ford, its products and Ford Motor Credit Company, please visit www.corporate.ford.com.

Ford of Europe is responsible for producing, selling and servicing Ford brand vehicles in 50 individual markets and employs approximately 45,000 employees at its wholly owned facilities and consolidated joint ventures and approximately 58,000 people when unconsolidated businesses are included. In addition to Ford Motor Credit Company, Ford Europe operations include Ford Customer Service Division and 18 manufacturing facilities (12 wholly owned facilities and six unconsolidated joint venture facilities). The first Ford cars were shipped to Europe in 1903 – the same year Ford Motor Company was founded. European production started in 1911.

| Contact: | Dan Jones |
| --- | --- |
|  | Ford of Europe |
|  | +44 (0) 1268 401917 |
|  | djone602@ford.com |